Environmental & Soil Sensors

1. Soil Moisture

Measurement Method: Time Domain Reflectometry (TDR) or Capacitance/FDR Sensors

- How it works: Measures the dielectric constant of soil. Water has a much higher dielectric constant (~80) than dry soil (~3-5). Sensors emit an electromagnetic signal and measure how quickly it returns or how much it's attenuated.

- Typical unit: Percentage (%) or volumetric water content (m³/m³)

- Your data range: 14.84% - 39.82% (seems reasonable)

- Placement: Inserted at root zone depth (15-30 cm)

2. Ambient Temperature

Measurement Method: Thermistor or Digital Temperature Sensor (DS18B20)

- How it works: Thermistors change resistance with temperature. Digital sensors convert temperature to a digital signal directly.

- Unit: Degrees Celsius (°C)

- Your data range: 20.20°C - 28.93°C

- Placement: Shielded from direct sunlight, at canopy height

3. Soil Temperature

Measurement Method: Temperature Probe with waterproof casing

- How it works: Similar to ambient sensors but designed for soil insertion. Often combined with moisture sensors.

- Unit: Degrees Celsius (°C)

- Your data range: 15.39°C - 22.78°C

- Placement: Same depth as soil moisture sensor

4. Humidity (Relative Humidity)

Measurement Method: Capacitive Humidity Sensor

- How it works: A polymer film absorbs/releases water vapor, changing its capacitance. Electronics convert this to %RH.

- Unit: Percentage (% RH)

- Your data range: 46.39% - 67.84%

- Placement: Near plant canopy, shaded

5. Light Intensity

Measurement Method: Photodiode or Solar Radiation Sensor (Pyranometer)

- How it works: Photodiodes generate current proportional to light intensity. More advanced sensors measure Photosynthetically Active Radiation (PAR, 400-700 nm).

- Unit: Lux or μmol/m²/s (PAR)

- Your data range: 241.41 - 596.14 (likely Lux or PAR units)

- Placement: At canopy level, facing upward

6. Soil pH

Measurement Method: pH Electrode/Ion-Selective Electrode (ISE)

- How it works: A glass electrode measures hydrogen ion concentration. Requires good soil contact and periodic calibration.

- Unit: pH units (0-14 scale)

- Your data range: 5.58 - 7.14 (slightly acidic to neutral)

- Placement: Inserted in moist soil, multiple readings averaged

Nutrient Sensors

7-9. Nitrogen, Phosphorus, Potassium Levels

Measurement Method: Ion-Selective Electrodes (ISE) or Optical Sensors

- Nitrogen (N): Usually measures nitrate (NO₃⁻) via nitrate ISE

- Phosphorus (P): Measures phosphate (PO₄³⁻) via phosphate ISE

- Potassium (K): Measures K⁺ ions via potassium ISE

- How ISE works: Membrane selective to specific ions creates voltage proportional to ion concentration

- Alternative: Soil spectroscopy - shine light on soil, measure reflected wavelengths to estimate nutrients

- Unit: mg/kg or ppm (parts per million)

- Your data range:

  - N: 10.00 - 30.71 ppm

  - P: 10.45 - 45.81 ppm

  - K: 17.94 - 39.93 ppm

- Challenge: Soil nutrients are heterogeneous; requires careful sampling

Plant Physiology Sensors

10. Chlorophyll Content

Measurement Method: SPAD Meter or NDVI/NDRE Sensors

- SPAD Meter: Clamp-on device that measures light transmission at 650 nm (chlorophyll absorbs) and 940 nm (reference)

- Optical Sensors: Measure Normalized Difference Vegetation Index (NDVI) - difference between near-infrared and red light reflection

- Unit: SPAD units or index (0-100)

- Your data range: 27.99 - 49.23 SPAD units

- Placement: Clamped on leaf or optical sensor above canopy

11. Electrochemical Signal (Most Advanced!)

Measurement Method: Microelectrodes or Surface Potential Sensors

- How it works:

  - Microelectrodes: Inserted into plant tissue (non-destructively) measure Action Potentials (APs) and Variation Potentials (VPs)

  - Surface Electrodes: Contact leaves/stems to measure bioelectric potentials

  - System measures: Voltage changes (mV or μV) in response to stimuli

- What it captures:

  - Action Potentials: Rapid electrical signals (seconds) traveling through phloem

  - Variation Potentials: Slower signals (minutes-hours) in response to wounds/stress

  - Spontaneous potentials: Baseline electrical activity

- Unit: Volts or arbitrary units (your data: 0.034 - 1.676)

- Placement: Microelectrodes in stem, or surface electrodes on leaves

- Challenges: Very sensitive to noise, requires shielding and amplification

Data Collection Challenges & Solutions:

| Feature | Challenge | Solution |
| --- | --- | --- |
| Soil Nutrients | Heterogeneous distribution | Multiple sensors, spatial averaging |
| Electrochemical | Signal noise (μV level) | Shielded cables, differential amplifiers |
| Chlorophyll | Leaf-to-leaf variation | Multiple leaves, consistent position |
| Soil Moisture | Calibration for soil type | Soil-specific calibration |
| All Sensors | Power management | Solar panels, low-power design |

The Fundamental Principle: Features Don't Work in Isolation

Plants are complex systems where every parameter affects others. Here's how they coordinate:

1. The "Conversation" Between Features

| Feature | What It "Talks About" | How It Coordinates |
| --- | --- | --- |
| Soil Moisture ↔ Electrochemical Signal | "I'm thirsty!" vs "I'm stressed!" | High EC signal + Low moisture = Definite water stress |
| Chlorophyll ↔ Nitrogen | "I can't make food!" vs "I have no nutrients!" | Low chlorophyll + Low N = Nutrient deficiency |
| Temperature ↔ Humidity | "I'm hot!" vs "I'm dry!" | High temp + Low humidity = Heat/transpiration stress |
| Light ↔ Chlorophyll | "Here's energy!" vs "I can use it!" | High light + Low chlorophyll = Wasted energy/potential |

2. Units and Their Meaningful Ranges

Let's break down each feature with units, ideal ranges, and what extremes mean:

A. Environmental Features

| Feature | Unit | Ideal Range | Too Low Means | Too High Means |
| --- | --- | --- | --- | --- |
| Soil Moisture | % VWC (Volumetric Water Content) | 20-35% (crop dependent) | Drought stress, wilting | Root rot, oxygen deprivation |
| Ambient Temp | °C | 20-30°C (most crops) | Stunted growth | Heat stress, wilting |
| Soil Temp | °C | 18-25°C | Slow germination | Root damage, poor absorption |
| Humidity | % RH | 50-70% | Water loss through leaves | Fungal diseases, poor transpiration |
| Light Intensity | Lux or μmol/m²/s | 10,000-50,000 lux | Poor growth | Photoinhibition, leaf burn |

B. Soil Chemistry Features

| Feature | Unit | Ideal Range | Too Low Means | Too High Means |
| --- | --- | --- | --- | --- |
| Soil pH | pH units | 6.0-7.0 (most crops) | Nutrient lock-up | Nutrient toxicity |
| Nitrogen (N) | ppm or mg/kg | 20-50 ppm | Chlorosis (yellow leaves) | Excessive foliage, poor fruiting |
| Phosphorus (P) | ppm or mg/kg | 10-30 ppm | Purple leaves, poor roots | Zinc/iron deficiency |
| Potassium (K) | ppm or mg/kg | 30-80 ppm | Weak stems, leaf edges brown | Magnesium deficiency |

C. Plant Physiology Features

| Feature | Unit | Ideal Range | Too Low Means | Too High Means |
| --- | --- | --- | --- | --- |
| Chlorophyll Content | SPAD units | 35-50 (most crops) | Nutrient deficiency | Very rare, maybe excessive N |
| Electrochemical Signal | mV or μV | 0.1-1.0 mV baseline | Healthy/resting state | Stress response, tissue damage |